| Reviewed on (Date): Reviewed by: | Reviewed on (Date): Reviewed by: | Reviewed on (Date): Reviewed by: | Reviewed on (Date): Reviewed by: |
| --- | --- | --- | --- |
| Approved on (Date): Approved by: | Approved on (Date): Approved by: | Approved on (Date): Approved by: | Approved on (Date): Approved by: |

| SR # | Process Name | Characteristics to be concerned | Standard Specifications |  | Person Responsible | Measuring Tool | Recorded on | Frequency |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Poly Lamination Process POLY LAMINATION PROCESS | 1- Poly Grammage | Poly grammage should be according to the limit defined in production orders Tolerance: + 2 |  | Operator | Weighing Scale | ## QRPOLYLAM/BUFC/07 | 1-Three-Times in a Shift FOR board having the grammage above 100gm/m2 2- Every reel for base Grammage less than 100g/m2 |
|  |  | 2-Wrinkles | Std: No Wrinkle Wrinkle Problem area of Reel will be identified with Tag |  | Operator | Visually | QRPOLYLAM/BUFC/ 01 | Every Reel |
|  |  | 3-Adhesion | Adhesion Limit should be according to the defined limit by QA Lab. Peel Strength 45g/cm min. or Fiber Picking as per visual standard | Operator |  | Peel strength Tester/ Manual | QRQA/BU-F/05 | 1-Three-Times in a Shift for board having the grammage above 100gm/m2 2- Every reel having the base grammage less than 100g/m2 |

| SR # | Process Name | Characteristics to be concerned | Standard Specifications | Person Responsible | Measuring Tool | Recorded on | Frequency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Poly Lamination Process | 4-Reel formation | According to the Shapes as defined in work instruction displayed in department. WI/POLLAM/BUFC/ 01 | Operator | Visually | WIPOLYLAM/BUFC/09 | Every Reel |
|  |  | 5-Un-Wind Brake Pressure | SP POLYLAM/BUFC/01 | ### Operator | Counting | ## QRPOLYLAM/BUFC/01 | Every Reel |
|  |  | 6-Rewinder Brake Pressure | SP POLYLAM/BUFC/01 | Operator | Pressure Gauge | QRPOLYLAM/BUFC/01 | Every Reel |
|  |  | 7- Secondary Un-Wind Pressure | SP POLLAM/BUFC/01 | Operator | Pressure Gauge | ## QRPOLYLAM/BUFC/01 | Every Reel |
|  |  | 8-Chilled water Temperature | SP POLLAM/BUFC/01 | Operator | Temperature Gauge | ## QRPOLYLAM/BUFC/ 01 | Every Reel |
|  |  | 9-Die,Barrel & Adopter Heaters Temperature | SP POLLAM/BUFC/03 | Operator | Temperature Controllers | QRPOLYLAM/BUFC/ 01 | Every Reel |

AMENDMENT HISTORY

| REV # | DATE | PART/ SECTION | NATURE OF AMENDMENT | DONE BY |
| --- | --- | --- | --- | --- |
| 3 | July 2024 | First page | Incorporation of review history | Aayesha |