| Reviewed on (Date): Reviewed by: | Reviewed on (Date): Reviewed by: | Reviewed on (Date): Reviewed by: | Reviewed on (Date): Reviewed by: |
| --- | --- | --- | --- |
| Approved on (Date): Approved by: | Approved on (Date): Approved by: | Approved on (Date): Approved by: | Approved on (Date): Approved by: |

| SR # | Process Name | Characteristics to be controlled | Standard Responsibilities | Person Responsible. | Measuring Tool | Recorded On | Frequency |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Printing Process | Text | Customer’s approved sample | Lead operator | Visually | ## QRLEMANIC/CL/02 | Once |
|  |  | Color Shade | Shade cards/ Customer’s approved sample/last bulk sample | Lead operator | Visually | ## QRLEMANIC/CL/12 | Every Pallet |
|  |  | Color Registration (Print to Print Registration) | ±0.15 mm | Lead operator | Measuring Strip | ## QRLEMANIC/CL/12 | Every Pallet |
|  |  | Varnish Viscosity | 20 to 22 sec | Lead operator | DIN CUP 4 & Stop Watch | ## QRLEMANIC/CL/12 QRLEMANIC/CL/02 | Every Pallet |
|  |  | Ink Viscosity | 18-20 sec | Lead operator | ZHAN CUP 4 & Stop Watch | ## QRLEMANIC/CL/02 | Once |
|  |  | Quality parameters i.e picking, printing lines etc | No defect | Lead operator | Visual | ## QRLEMANIC/CL/02 | Every Pallet |
| 2 | Cutting & Creasing Process | Cutting | Cutting lines should be fully cut | Lead operator | Visually | ## QRLEMANIC/CL/12 | Every Pallet |
|  |  | Creasing | Creases should not crack while folding | Lead operator | Visually | ## QRLEMANIC/CL/12 | Every Pallet |
|  |  | Print-to-cut registration (Longitudinal + Lateral) | ±0.5 mm | Lead operator | Measuring Strip | ## QRLEMANIC/CL/12 | Every Pallet |
|  |  | Print-to-Emboss registration (Longitudinal + Lateral)- Rotary | ±0.2 mm | Lead operator | Measuring Strip | ## QRLEMANIC/CL/12 | Every Pallet |
|  |  | Print-to-Emboss registration (Longitudinal + Lateral)- Flat bed | ±0.5 mm | Lead operator | Measuring Strip | ## QRLEMANIC/CL/12 | Every Pallet |
|  |  | Cutting Temperature | 34-40 ˚C | Lead operator | Temperature meter | ## QRLEMANIC/CL/02 | Once |
| 3 | General | Board grammage | Job card | Lead operator | - | ## QRLEMANIC/CL/02 | Once |
|  |  | No. Of ups | Job card | Lead operator | Visual counting | ## QRLEMANIC/CL/02 | Once |

AMENDMENT HISTORY

| REV # | DATE | PART/ SECTION | NATURE OF AMENDMENT | DONE BY |
| --- | --- | --- | --- | --- |
| 11 | July 2024 | First page | Incorporation of review history | Aayesha |